SENSORA

Hardware Feature Build Checklist

Worker ESP32 + Wall ESP32 — features to implement one at a time, checked off as you go

Prepared 16 September 2026

Reality check —  Feature 1 is a genuine ~2-hour job if your ESP-NOW link between the two boards already passes a test packet. Feature 2 (full LiDAR-style mapping) is not a 2-hour feature — MPU6050 has no magnetometer, so a gyro-only yaw estimate drifts within minutes, and a real point-cloud needs a servo to sweep the sensor; it's scoped down below to something genuinely buildable today. Feature 3 (in/out counting) needs hardware you don't have yet — a single sensor detects presence, not direction — so the real version is written for when you get it, with no shortcut pretending otherwise.

# Feature 1 — Fall / Inactivity Detection → Distress Signal

## Build this first.   (~2 hours)

- Confirm the ESP-NOW link between the worker and wall ESP32 already sends/receives a basic test packet — fix this first if it doesn't, everything else depends on it

- Wire MPU6050 SDA/SCL to the worker ESP32's I²C pins (GPIO21/22), power from 3.3V

- Install Adafruit_MPU6050 + Adafruit_Sensor (or MPU6050_light) via Library Manager

- Write a throwaway sketch that prints accel X/Y/Z at ~10Hz to confirm wiring is good

- Every loop, compute a rolling “motion energy” value (e.g. sum of |accel − lastAccel| over the last second)

- Define INACTIVITY_THRESHOLD and INACTIVITY_DURATION_SEC as named constants at the top of the file, don't bury them in logic

- Run a millis()-based timer that resets to zero every time motion energy exceeds the threshold

- When the timer exceeds INACTIVITY_DURATION_SEC with no reset, set local state to FALL_SUSPECTED

- The instant FALL_SUSPECTED triggers, send an ESP-NOW packet to the wall node: {workerId, msgType=DISTRESS, timestamp}

- Also flash an LED / sound a buzzer on the worker board itself the instant it triggers — this must work even if the packet never arrives

- On the wall ESP32, register a receive callback that checks msgType==DISTRESS and immediately writes it as a JSON line over Serial

- On the laptop, run the simplest possible Python script that reads that serial line and prints it, so you can see the event land

- Temporarily set INACTIVITY_DURATION_SEC to ~10s while testing so you're not waiting a full minute per test; set it back before demoing

→ Dashboard, later: a worker card that flips red and shows “FALL SUSPECTED — [time]” the moment a DISTRESS message hits the server

# Feature 2 — Proximity Snapshot at the Moment of Distress

A scoped-down version of the LiDAR idea. A live sweeping map is a multi-session feature, not a 2-hour one — this version captures useful spatial context at the exact moment something goes wrong, which is the actual value of the idea, without requiring a servo or a magnetometer you don't have.

## Do this second.   (~2 hours)

- Wire an HC-SR04 (or similar) ultrasonic sensor's TRIG/ECHO to two free GPIOs on the worker board

- Put a voltage divider (e.g. 1kΩ/2kΩ) on the ECHO line — HC-SR04's echo swings to 5V, above ESP32's 3.3V logic max

- Write a test function that pulses TRIG and times ECHO to get distance in cm; confirm it reads ~30cm when 30cm from a wall

- Reuse the MPU6050 already wired for Feature 1 to integrate a yaw estimate: yaw += gyroZ * dt, reset to 0 at boot

- Accept and label this yaw as “rough direction only” — it will drift over minutes with no magnetometer to correct it, don't present it as a compass heading

- On every FALL_SUSPECTED or SOS trigger, take one ultrasonic reading + the current yaw value and add them as extra fields on the DISTRESS packet (distanceCm, yawDeg)

- Test by triggering SOS at a known distance and orientation from a wall and checking the received numbers roughly match

- Explicitly defer continuous sweeping/point-cloud mapping — note it as a future feature needing a servo (SG90) to rotate the sensor, not part of today's build

→ Dashboard, later: a small directional arrow + distance readout on the active alert card, not a full map

# Feature 3 — Worker In/Out Counting at the Wall Node

## Needs new hardware before you can start.

- You can't build this with what you have right now — one ultrasonic/IR sensor tells you something passed, not which way; you need two beam sensors ~30–50cm apart at a single choke point to infer direction from which one triggers first

- Once you have two IR break-beam sensors (or a second ultrasonic), wire both to the wall ESP32's digital inputs

- Test each sensor independently — confirm it flips state when you wave a hand through it

- Direction logic: sensor A triggers before B within ~1.5s → IN; B before A → OUT

- Maintain a workerCount int on the wall node, increment on IN, decrement on OUT (floor at 0)

- Send workerCount to the server on every change, plus every ~30s as a heartbeat so the dashboard isn't stale

- Walk the gate both directions a few times and confirm the count tracks correctly; widen the timing window if a slow walk-through gets missed

- Don't try to fake this with RSSI/association timing from the existing mesh — direction is fundamentally ambiguous without two sensors, it's not worth the debugging time

# Other Basic Features Worth Adding

In rough priority order — each is small enough to slot into a spare hour once the three features above are working.

- SOS/panic button — one GPIO, immediate DISTRESS packet, always overrides every other state

- Local buzzer + status LED — shows NORMAL/WARNING/CRITICAL/SOS even with zero network, don't make this depend on the mesh working

- Battery voltage monitoring — ADC + voltage divider, send battery % every packet, flag LOW_BATTERY below a threshold

- Worker heartbeat packet — send a small “I'm alive” every N seconds so a worker can be marked OFFLINE (not silently NORMAL) if packets stop

- Wall node → server link watchdog — a distinct LED pattern on the wall ESP32 if its own serial/server link drops, so a backend disconnect is visible at the table

- Hardcoded WORKER_ID per device — so the server labels events by person, not raw MAC address

- Basic temperature/gas sensing (whatever you already have wired) — worker state shouldn't be motion-only

- Timestamped event log on the server (even a flat file or SQLite row per event) — this is what the dashboard's event timeline actually reads from